## Speedo Gear Selection Many of you have changed tire size or axle ratio on your car and now the speedo reads too high or low. While this can be corrected with an aftermarket ratio adapter, I like to change the speedometer driven gear at the rear of the tranny to correct for error. If you want "correct", this is the way to go. #### Determining Speedo Error Checking the accuracy of your speedometer is easy. Look for a stretch of interstate highway with mile markers. Use a stop watch (or a watch with a sweep second hand) to determine the time to travel one mile with your speedo indicating exactly 60 MPH. Let's say that you came up with 69.2 sec. Now find your actual MPH using this formula: Actual MPH = 3600/Time (Time in seconds to travel one mile at 60 MPH indicated) Example: 3600/69.2 = 52 MPH actual, an error of 8 MPH Percent Error = MPH error/actual MPH (8/52 = 15.4%) #### Correcting Error Turbo HydraMatic 400: Speedo driven gears have from 34 to 45 teeth. Each additional tooth lowers the speedo reading about 2.5%. Each tooth removed raises the speedo reading about 2.5%. In our example, we are reading 15.4% high so we would need to add teeth to spin the speedo cable slower. Remember, one tooth = 2.5% so we need to add about six teeth. (15.4%/2.5% = 6.2 teeth). Turbo HydraMatic 350, Powerglide & 4 Speed: Speedo driven gears have from 18 to 25 teeth. Each additional tooth lowers the speedo reading about 5%. Each tooth removed raises the speedo reading about 5%. In our example, we are reading 15.4% high so we would need to add teeth to spin the speedo cable slower. Remember, one tooth = 5% so we need to add about three teeth. (15.4%/5% = 3.1 teeth). At this point, pull the speedo gear out of your tranny by removing the small bolt and clamp that retains the speedo gear sleeve and pulling on the speedo cable. Then count the teeth, subtract or add to the number of teeth and select a new gear from the chart below. Use color only as a check. Gears immersed in tranny fluid for many years will be stained and off color. Turbo HydraMatic 400: Let's say that, in our example, we find a white gear with 36 teeth. Adding six teeth to correct a speedo reading 15% low gives us a 42 teeth. We need to buy a 42 tooth (green) 1362049 gear. Turbo HydraMatic 350, Powerglide & 4 Speed: Let's say that, in our example, we find a blue gear with 20 teeth. Adding three teeth to correct a speedo reading 15% low gives us a 23 teeth. We need to buy a 23 tooth (black) 3860346 gear. If you find that your "correct" gear falls between two tooth counts as it does in this case, it's usually best to pick the smaller gear so your speedo will read slightly high. As I recall, GM practice was to pick speedo gears so that the speedo would read between 60 and 63 MPH at a true 60 MPH. Most of these parts are still available from GM so see your local dealer for parts. They'll have the best prices and you'll find most very cooperative if you know which Part Numbers you need. Most won't be very helpful, however, if you ask them to solve your problem. Takes a lot of effort and risk on their part as they're not usually very familiar with the 60's and 70's products and, of course, you'll want them to take the parts back if they're the wrong stuff and then they'll be stuck with parts they'll probably never sell.

| TH400 Driven Gears. Part Number; Teeth; Color Code; Sleeve. 9774413; 34; Lt. Green; 1362294. 9780628; 35; Pink; 1362294. 1359270; 36; White; 1362294. 1359271; 37; Red; 1362294. 1359272; 38; Blue; 1362294. 1359273; 39; Brown; 1362294. 1362048; 40; Black; 1362293. 1362195; 41; Yellow; 1362293. 1362049; 42; Green; 1362293. 1362196; 43; Purple; 1362293. 9780470; 44; Dk. Gray; 1362293. 9775187; 45; Lt. Blue; 1362293. |  | TH350, 4 Speed & Powerglide Driven Gears. Part Number; Teeth; Color Code; PG Sleeve; 350/4SP Sleeve. 3987917; 17; Purple; 3969914; 345215. 3987918; 18; Brown; 3969914; 345215. 3987919; 19; Natural; 3969914; 345215. 3987920; 20; Blue; 3969914; 345215. 3987921; 21; Red; 3969914; 345215. 3987922; 22; Silver; 3969914; 345215. 3860345; 22; Green; 3969914; 345215. 3860346; 23; Black; 3969914; 345215. 3860347; 24; Yellow; 3969914; 345215. 3860348; 25; Orange; 3969914; 345215. |
| --- | --- | --- |

#### Speedo Sleeves GM calls the part that mounts the speedo driven gear to the tranny a Sleeve - Speedo Driven Gear. The THM400 uses two sizes which take care of the different diameters of the various driven gears. They are marked with the driven gear tooth numbers they will accept. In our example, our white gear used a PN 1362294 sleeve so we have to buy a new sleeve (PN 1362293) to match our new 42 tooth greengear. #### Off the Chart or Can't Buy the Correct Gear Ahh, back where we started. We'll have to use a ratio adapter and perhaps a speedo gear change as well to tune in the speedo. GM used #3932220 ratio adapter on many Chevelles. This part has a ratio of about 1.35/1 and retails for about $45 from your friendly GM dealer. By the Numbers If you are into formulas, you can calculate the correct gear as follows: - Count the teeth on the speedo drive gear, the one mounted on the transmission output shaft - Determine your axle ratio - Compute the number of teeth on the correct driven gear with this formula N(driven) = N(drive) x (Axle Ratio) x (Rev per Mile) /1001 If this comes out as something other than a whole number (as it usually will), then go on the small side to keep speedo reading slightly higher than actual. Principal here is that a correctly calibrated GM mechanical speedo turns 1001 rpms at 60 mph.