Rear power window safety switch

If your tailgate window rolls down on your blazer and does not roll up, it's most likely your window safety switch. It's located on driver side tailgate latch. Inside the switch are copper contacts that get dirty and corrode. There is no need for a jumper wire or new switch; all you need to do is clean the contacts with small wire brush.

1- lower tailgate, take screws out of the tailgate plate in the center of tailgate.
Unhook latch control rod from clip on latch control in center of tailgate on driver side.

2- unscrew tailgate latch on driver side and carefully pull latch and switch out of tailgate.
Unhook electric plug from switch and unbolt switch from latch.

3- you need a small flat head screwdriver to pry up 4 metal tabs on the switch top.
Carefully remove the cover from metal bottom. There is a spring and small copper square piece under pressure.

4- you need to clean the 3 half round copper contacts and square copper piece with a small wire brush.

5- check your springs and change if needed.

6- put some dielectric grease on all copper contacts, put switch back together and bend tabs back with pliers.

Reassemble.